Prime Logic CT5 Specification Sheet (updated 3-11-19)

Technical Specifications

Axle to Axle;		35" +/- .125"		Flex Guard Spring;		3 1/2" (2 dimples)

Timing;			Timing Dots Even		Cam Spacers;			.080" (+/- .005" to fit)

Brace Height;		6" +/-.125"		String Stop Length;		6 1/8”

Centershot;		.635”-.764”		Cam lean;			.150" Max

Planarity;		.743” +/- .090"

Cam System

TRM (Draw lengths are minus nothing, plus 1/4". All cams have 80% letoff)

| CAM NUMBER | DRAW LENGTH | ATA RATING* |
| --- | --- | --- |
| #1 | 30" | 332-328 |
| #2 | 29 1/2" | 328-324 |
| #3 | 29" | 324-320 |
| #4 | 28 1/2" | 320-316 |
| #5 | 28" | 316-312 |
| #6 | 27 1/2" | 312-308 |
| #7 | 27" | 308-304 |
| #8 | 26 1/2" | 304-300 |
| #9 | 26" | 300-296 |
| #10 | 25 1/2" | 296-292 |
| #11 | 25" | 292-288 |
| #12 | 24 1/2" | 288-284 |

*Estimated performance at 70lbs, 350 grains.

String Specifications

| Serving Locations |  |  |  |  |  |  |  | Overall Length |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| String | 1 1/2" | 10 1/4" | 14 1/4" | 17 1/2" | 19 1/2” | 25 7/8” | - | 28.45” |
| Cable | 12” | 16 3/8” | 28 13/16” | 34 15/16” | - | - | - | 41.4” |
| Upper Yoke | 16” | 16 5/8" | 18 1/8" | 18 3/4” | - | - | - | 34.75” |
| Lower Yoke | 14 1/2" | 15 1/8” | 16 5/8” | 17 1/4" | - | - | - | 31.8” |

Limb Deflections

| Nominal Defl. | Defl. Range | Draw Weight |
| --- | --- | --- |
| 56 | 53-59 | 81-83 lbs |
| 62 | 59-65 | 71-73 lbs |
| 67 | 64-70 | 66-68 lbs |
| 72 | 69-75 | 61-63 lbs |
| 84 | 81-87 | 51-53 lbs |
| 102 | 99-105 | 41-43 lbs |